Heritage Impact Assessment: Land adjacent and south of Watcombe Hall

| HELAA2021 Ref: | 24T012 |
| --- | --- |
| Site Address: | Land adjacent and south of Watcombe Hall |
| Site Area (ha): | 3.89 |
| Site Yield: |  |
| HIA Assessment |  |

Site overview and heritage context:

- Steeply sloped

- Wooded area

- semi-managed with significant tree cover and managed pathways

Green space sloping down towards Watcombe Hall and Washington House to the north, and Watcombe Beach to the east. The site is semi-managed with significant tree cover and managed pathways. It is bounded on all sides by mature vegetation.

The northeastern section of the site is within the Watcombe Park Conservation Area and the Grade II listed building ‘Washington House’ is located around 30m to the north of the site.

Planning History:

P/2021/1373 – Construction of 9 detached homes on eastern part of site – Approved 21/10/2022

| Heritage Asset Affected by Allocation | Designation | Contributing Elements to Significance | Impact of Development on Significance | Mitigation Measures for any Identified Harm /Opportunities to Enhance Significance | Impact on Significance with Mitigation in place |
| --- | --- | --- | --- | --- | --- |
| Watcombe Park Conservation Area Washington House (N of site) Lantern House, Watcombe Lodge (NW of site) | Conservation Area Grade II listed building Grade II listed building | The conservation area has demonstrable historic value through its association with Brunel, architectural value through the surviving historic built environment and artistic value through its designed landscape in association with Brunel Manor. The site is a large area of open space which forms part of the rural character of the area and makes a positive contribution to the significance of the asset. Shown on OS map as Watcombe House. Villa. 1858. Forms part of a loose-knit group of early-mid 19th century villas, some with service wings, to the southeast of Watcombe Park. The site forms part of the rural setting of the asset which makes a positive contribution to its significance. Villa, divided into two. c1860, on the east side of Teignmouth Road, built into the hillside. Forms part of a loose-knit group of early-mid 19th century villas, some with service wings, to the southeast of Watcombe Park. The site forms part of the rural setting of the asset which makes a positive contribution to its significance. | The development of the site has the potential have a detrimental impact on the existing open space within the site and would dilute its rural character. The level of impact would depend on the proposed quantum of development. It would also detrimentally impact on the historic pattern of development within this part of the conservation area which is for large villas located within substantial plots. This would cause less than substantial harm to the significance of the conservation area. The development of the site has the potential have a detrimental impact on the existing open space within the site and would dilute its rural character. The level of impact would depend on the proposed quantum of development. It would also detrimentally impact on the historic pattern of development within this part of the conservation area which is for large villas located within substantial plots. This would cause less than substantial harm to the significance of the conservation area. The site is within the wider setting of the Listed Building and has the potential to impact on its setting, however this is unlikely to be to any measurable degree. | Development of the site with well-considered placement of dwellings set within substantial grounds would retain the rural character of the site could reduce the level of harm to result in a neutral impact. Restricting development to outside the conservation area would also help lessen impact. Development of the site with well-considered placement of dwellings set within substantial grounds would retain the rural character of the site could reduce the level of harm to result in a neutral impact. Restricting development to a suitable distance from the Listed Building and a considered landscaping and planting scheme may also lessen impact on setting. n/a | Neutral Neutral Neutral |